Prepared By:

Ahmed Sabry Mohamed El-Bayoumi

Ebrahem Mohamed Abdelmageed Hassan

Mohamed Ayman Nabwy Abd El-Maksoud

Mohamed Elsayed Ibrahem Sayed

Shady Hossam Eissa Abdelkaleq

Supervised By:

Eng. Sameh Kamal El-shamy

Course:

Cisco Network Administrator

Content:

- Introduction

- Team Responsibilities

- Project Overview

- Network Topology

- Cairo (HQ)

- Paris (Branch)

- ISP Connectivity

- WE ISP

- Vodafone ISP

- VPN Configuration

- Servers Configuration

- Security Configuration

- Configuration:

Servers

- Web server

- DNS server

- FTP server

- Email server

- RADIUS & AAA server

- NTP & Syslog server

Basic configuration (Routers & Switches):

- Banner

- Line console

- Line VTY (SSH)

- DHCP on SW L3

- VTP

Advantage configurations:

- EtherChannel

- IVLAN

- HSRP

- OSPF

- STP

- Wireless Configurations

Security configurations:

- L2 security configurations

- NAT

- VPN

- ACL

- Data Sheets

- Head Quarter

- Routers

- L3 Switches

- L2 Switches

- Paris Branch

- Routers

- L3 Switches

- L2 Switches

- Conclusion

## Introduction

This document provides a detailed overview of the project undertaken to interconnect two branches of our company: the Cairo branch and the Paris branch. The objective is to establish a reliable, secure, and efficient network connection between the two locations using dual Internet Service Providers (ISPs) and Virtual Private Network (VPN) technologies. This setup ensures redundancy, load balancing, and secure data transmission between the branches.

## Project Overview

The project involves connecting the Cairo branch, consisting of five floors, with the Paris branch, comprising six floors. Both branches are interconnected via two ISPs, WE and Vodafone, ensuring high availability and redundancy. Additionally, a VPN setup is implemented to secure the data transmission between the branches. Each floor within the branches is segmented into specific VLANs to optimize network performance and security.

## Team Responsibilities

### Ahmed Sabry

- Responsibilities:

  - DHCP on Layer 3 Switches

  - VPN Configuration

  - Web Server Configuration

  - FTP Server Configuration

  - Email Server Configuration

  - RADIUS & AAA Server Configuration

  - NTP & Syslog Server Configuration

  - DNS Server Configuration

### Ebrahim Mohamed

- Responsibilities:

  - Basic Configuration of Routers & Switches:

    - Host Name Configuration

    - Banner MOTD Configuration

    - Line Console Configuration

    - Line VTY Configuration

    - VTP Configuration

    - DTP Configuration

### Shady Hossam

- Responsibilities:

  - Advanced Configurations

    - EtherChannel Configuration

    - HSRP Configuration

    - OSPF Configuration

    - STP Configuration

### Mohamed Nabawy

- Responsibilities:

  - Security Configurations:

    - Layer 2 Security Configurations

    - NAT Configuration

    - ACL Configuration

    - Static & Default Route Configuration

### Mohamed Elsayed

- Responsibilities:

  - Planning and Designing

  - Subnetting and IP Addressing

  - IVLAN Configuration

  - Wireless Configurations

  - Ensure All Deliverables

## Network Topology

### Cairo Branch

- Structure:

  - Number of Floors: 5

  - Switches:

    - Layer 2 Switches: one per floor

    - Layer 3 Switches: Two, connected to the Layer 2 switches

  - Routers: Two, each connected to a Layer 3 switch

  - ISPs: Connected to each router (WE and Vodafone)

- VLANs:

  - VLAN 10: HR

  - VLAN 20: Sales

  - VLAN 30: IT

  - VLAN 40: CEO

  - VLAN 50: Finance

  - VLAN 99: Native

  - VLAN 100: Management (MGMT)

  - VLAN 150: VOIP

  - VLAN 160: WLAN

  - VLAN 170: Printer

  - VLAN 200: Unused

### Paris Branch

- Structure:

  - Number of Floors: 6

  - Switches:

    - Layer 2 Switches: one per floor

    - Layer 3 Switches: Two, connected to the Layer 2 switches

  - Routers: Two, each connected to a Layer 3 switch

  - ISPs: Connected to each router (WE and Vodafone)

- VLANs:

  - VLAN 10: PR

  - VLAN 20: C&S

  - VLAN 30: SE

  - VLAN 40: IT

  - VLAN 50: HR

  - VLAN 60: R&D

  - VLAN 99: Native

  - VLAN 150: VOIP

  - VLAN 160: WLAN

  - VLAN 170: Printer

  - VLAN 100: Management (MGMT)

  - VLAN 200: Unused

## VLAN Configuration

### Cairo Branch VLANs

| VLAN ID | Name | Description |
| --- | --- | --- |
| 10 | HR | Human Resources |
| 20 | Sales | Sales Department |
| 30 | IT | Information Technology |
| 40 | CEO | Chief Executive Officer |
| 50 | Finance | Finance Department |
| 99 | Native | Native VLAN for untagged traffic |
| 100 | Management | Network Management |
| 150 | VOIP | Voice Over IP |
| 160 | WLAN | Wireless LAN |
| 170 | Unused | Printer |
| 200 | Unused | Reserved for future use |

### Paris Branch VLANs

| VLAN ID | Name | Description |
| --- | --- | --- |
| 10 | PR | Public Relations |
| 20 | C&S | C&S Department |
| 30 | SE | Software Engineering |
| 40 | IT | Information Technology |
| 50 | HR | Human Resources |
| 60 | R&D | Research and Development |
| 99 | Native | Native VLAN for untagged traffic |
| 100 | Management | Network Management |
| 150 | VOIP | Voice Over IP |
| 160 | WLAN | Wireless LAN |
| 170 | Unused | Printer |

## ISP Connectivity

### WE ISP

- Connection Details:

  - Link Type: Fiber Optic

  - Redundancy: Dual ISP setup with Vodafone

- Connections:

  - Cairo Branch: Connected to both Layer 3 routers

  - Paris Branch: Connected to both Layer 3 routers

### Vodafone ISP

- Connection Details:

  - Link Type: DSL

  - Redundancy: Dual ISP setup with WE

- Connections:

  - Cairo Branch: Connected to both 2 routers

  - Paris Branch: Connected to both 2 routers

### Server Connectivity

Both ISPs are connected to three key servers:

| Server | Description |
| --- | --- |
| FACEBOOK | Facebook Recognition Server |
| YOUTUBE | YouTube Streaming Server |
| TWITTER | Twitter Integration Server |

## VPN Configuration

A Virtual Private Network (VPN) is established between the Cairo and Paris branches to ensure secure communication over the internet. The VPN setup includes:

- VPN Type: Site-to-Site GRE VPN

- Redundancy: Dual ISP connections to ensure high availability

- Routing Protocol: OSPF for dynamic routing over the VPN

## Server Configuration

### Web Server

- Function: Hosts the company’s official website.

- Configuration: Secured with SSL certificates, accessible via HTTPS.

### FTP Server

- Function: Facilitates file transfers within the organization.

- Configuration: Configured with user authentication and access controls.

### Email Server

- Function: Manages company email communications.

- Configuration: Integrated with Active Directory for user management.

### RADIUS & AAA Server

- Function: Handles authentication, authorization, and accounting for network access.

- Configuration: Integrated with network devices for secure access control.

### NTP & Syslog Server

- Function: Synchronizes network device clocks and logs system events.

- Configuration: Configured to provide accurate time stamps and centralized logging.

## Security Configurations

### Layer 2 Security Configurations

#### DHCP Snooping

- Description: Protects against rogue DHCP servers by allowing only trusted DHCP messages.

- Configuration:

#### ARP Inspection

- Description: Prevents ARP spoofing by validating ARP packets against DHCP Snooping Binding Table.

- Configuration:

### NAT (Network Address Translation)

- Description: Translates private IP addresses to public IP addresses for internet access.

- Configuration:

### VPN (Virtual Private Network)

- Description: Secures data transmission between Cairo and Paris branches.

### ACL (Access Control Lists)

- Description: Controls traffic flow and restricts unauthorized access.

Configuration

1.Web server

2.DNS server

3.FTP server

4.Email server

5.RADIUS & AAA server

6.NTP & Syslog server

WLC

HQ_DS_sw-1

| - Host name hostname HQ_DS_sw_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Trunk, Native Interfaces int range g1/0/5-10 no sh switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan all int g1/0/12 no switchport ip add 172.16.1.5 255.255.255.252 interface GigabitEthernet1/0/20 switchport trunk native vlan 100 switchport mode trunk - IVLAV , HSRP , Helper-address int vlan 10 description Default Gateway SVI for HR Team ip add 172.16.10.2 255.255.255.0 standby 10 ip 172.16.10.1 standby 10 priority 200 standby 10 preempt ip helper-address 172.16.1.19 no sh int vlan 20 description Default Gateway SVI for Sales Team ip add 172.16.20.2 255.255.255.0 standby 20 ip 172.16.20.1 standby 20 priority 200 standby 20 preempt ip helper-address 172.16.1.19 no sh int vlan 30 description Default Gateway SVI for IT Team ip add 172.16.30.2 255.255.255.0 standby 30 ip 172.16.30.1 standby 30 priority 200 standby 30 preempt ip helper-address 172.16.1.19 no sh int vlan 100 description Default Gateway SVI for Management Team ip add 172.16.100.2 255.255.255.0 standby 100 ip 172.16.100.1 standby 100 priority 200 standby 100 preempt ip helper-address 172.16.1.19 no sh int vlan 40 description Default Gateway SVI for CEO Team ip add 172.16.40.2 255.255.255.0 standby 40 ip 172.16.40.1 standby 40 preempt ip helper-address 172.16.1.19 no sh int vlan 50 description Default Gateway SVI for Finance Team ip add 172.16.50.2 255.255.255.0 standby 50 ip 172.16.50.1 standby 50 preempt ip helper-address 172.16.1.19 no sh int vlan 150 description Default Gateway SVI for VIOP devices ip add 172.16.150.2 255.255.255.0 standby 150 ip 172.16.150.1 - FTP service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - DHCP ip dhcp pool HR network 172.16.10.0 255.255.255.0 default-router 172.16.10.1 dns-server 172.16.5.18 ip dhcp pool Sales network 172.16.20.0 255.255.255.0 default-router 172.16.20.1 dns-server 172.16.5.18 ip dhcp pool IT network 172.16.30.0 255.255.255.0 default-router 172.16.30.1 dns-server 172.16.5.18 ip dhcp pool CEO network 172.16.40.0 255.255.255.0 default-router 172.16.40.1 dns-server 172.16.5.18 ip dhcp pool Finance network 172.16.50.0 255.255.255.0 default-router 172.16.50.1 dns-server 172.16.5.18 ip dhcp pool MGMT network 172.16.100.0 255.255.255.0 default-router 172.16.100.1 dns-server 172.16.5.18 ip dhcp pool Voip network 172.16.150.0 255.255.255.0 default-router 172.16.150.1 dns-server 172.16.5.18 ip dhcp pool Wlan network 172.16.160.0 255.255.255.0 default-router 172.16.160.1 | - VTP vtp version 2 vtp domain cisco.com vtp mode server vtp password cisco vlan 10 name HR vlan 20 name Sales vlan 30 name IT vlan 40 name CEO vlan 50 name Finance vlan 100 name MGMT vlan 150 name VIOP vlan 160 name Wlan vlan 170 name Printer vlan 99 name Native vlan 200 name Unused - STP spanning tree mode pvst spanning tree vlan 10,20,30,100 root primary spanning tree vlan 40,50,150,160,170 root secondary - Ether-channel int range g1/0/1-4 no switchport channel-group 10 mode active no sh int port-channel 10 ip add 172.16.1.1 255.255.255.252 switchport trunk native vlan 99 no sh - Unused Interfaces int range g1/0/11-19,g1/0/21-24, g1/1/1-4 sh switchport mode access switchport access vlan 200 standby 150 preempt ip helper-address 172.16.1.19 no sh int vlan 160 description Default Gateway SVI for Wlan devices ip add 172.16.160.2 255.255.255.0 standby 160 ip 172.16.160.1 standby 160 preempt ip helper-address 172.16.1.19 no sh int vlan 170 description Default Gateway SVI for Printer devices ip add 172.16.170.2 255.255.255.0 standby 170 ip 172.16.170.1 standby 170 preempt ip helper-address 172.16.1.19 no sh dns-server 172.16.5.18 ip dhcp pool Printer network 172.16.170.0 255.255.255.0 default-router 172.16.170.1 dns-server 172.16.5.18 ip dhcp excluded-address 172.16.10.1 172.16.10.49 ip dhcp excluded-address 172.16.10.125 172.16.10.255 ip dhcp excluded-address 172.16.20.1 172.16.20.49 ip dhcp excluded-address 172.16.20.125 172.16.20.255 ip dhcp excluded-address 172.16.30.1 172.16.30.49 ip dhcp excluded-address 172.16.30.125 172.16.30.255 ip dhcp excluded-address 172.16.40.1 172.16.40.49 ip dhcp excluded-address 172.16.40.125 172.16.40.255 ip dhcp excluded-address 172.16.50.1 172.16.50.49 ip dhcp excluded-address 172.16.50.125 172.16.50.255 ip dhcp excluded-address 172.16.100.1 172.16.100.49 ip dhcp excluded-address 172.16.100.125 172.16.100.255 ip dhcp excluded-address 172.16.150.1 172.16.150.49 ip dhcp excluded-address 172.16.150.125 172.16.150.255 ip dhcp excluded-address 172.16.160.1 172.16.160.49 ip dhcp excluded-address 172.16.160.125 172.16.160.255 ip dhcp excluded-address 172.16.170.1 172.16.170.49 ip dhcp excluded-address 172.16.170.125 172.16.170.255 |
| --- | --- |

HQ_DS_sw_2

| - Host name hostname HQ_DS_sw_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - -Trunk Interfaces int range g1/0/5-10 no sh switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan all int g1/0/12 no switchport ip add 172.16.1.13 255.255.255.252 - Unused Interfaces int range g1/0/11-24, g1/1/1-4 sh switchport mode access switchport access vlan 200 - STP spanning tree mode pvst spanning tree vlan 40,50,150,160,170 root primary spanning tree vlan 10,20,30,100 root secondary - IVLAV , HSRP , Helper-address int vlan 10 description Default Gateway SVI for HR Team ip add 172.16.10.3 255.255.255.0 standby 10 ip 172.16.10.1 standby 10 preempt ip helper-address 172.16.1.19 no sh int vlan 20 description Default Gateway SVI for Sales Team ip add 172.16.20.3 255.255.255.0 standby 20 ip 172.16.20.1 standby 20 preempt ip helper-address 172.16.1.19 no sh int vlan 30 description Default Gateway SVI for IT Team ip add 172.16.30.3 255.255.255.0 standby 30 ip 172.16.30.1 standby 30 preempt ip helper-address 172.16.1.19 no sh int vlan 100 description Default Gateway SVI for Management Team ip add 172.16.100.3 255.255.255.0 standby 100 ip 172.16.100.1 standby 100 preempt ip helper-address 172.16.1.19 no sh int vlan 40 description Default Gateway SVI for CEO Team ip add 172.16.40.3 255.255.255.0 standby 40 ip 172.16.40.1 standby 40 priority 200 standby 40 preempt ip helper-address 172.16.1.19 no sh int vlan 50 description Default Gateway SVI for Finance Team ip add 172.16.50.3 255.255.255.0 standby 50 ip 172.16.50.1 standby 50 priority 200 standby 50 preempt ip helper-address 172.16.1.19 no sh int vlan 150 description Default Gateway SVI for VIOP devices ip add 172.16.150.3 255.255.255.0 standby 150 ip 172.16.150.1 standby 150 priority 200 standby 150 preempt - OSPF , Static Routing ip routing router ospf 10 router-id 2.2.2.2 passive-interface GigabitEthernet1/0/6 passive-interface GigabitEthernet1/0/7 passive-interface GigabitEthernet1/0/8 passive-interface GigabitEthernet1/0/9 passive-interface GigabitEthernet1/0/10 passive-interface GigabitEthernet1/0/20 network 172.16.1.0 0.0.0.3 area 0 network 172.16.10.0 0.0.0.255 area 0 network 172.16.20.0 0.0.0.255 area 0 network 172.16.30.0 0.0.0.255 area 0 network 172.16.40.0 0.0.0.255 area 0 network 172.16.50.0 0.0.0.255 area 0 network 172.16.100.0 0.0.0.255 area 0 network 172.16.150.0 0.0.0.255 area 0 network 172.16.160.0 0.0.0.255 area 0 network 172.16.1.12 0.0.0.3 area 0 network 172.16.1.28 0.0.0.3 area 0 network 172.16.170.0 0.0.0.255 area 0 ip route 172.16.5.34 255.255.255.255 172.16.5.30 ip route 172.16.5.35 255.255.255.255 172.16.5.30 - FTP service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 | - VTP vtp version 2 vtp domain cisco.com vtp mode server vtp password cisco vlan 10 name HR vlan 20 name Sales vlan 30 name IT vlan 40 name CEO vlan 50 name Finance vlan 100 name MGMT vlan 150 name VIOP vlan 160 name Wlan vlan 170 name Printer vlan 99 name Native vlan 200 - Ether-channel int port-channel 10 ip add 172.16.1.2 255.255.255.252 no sh int range g1/0/1-4 no switchport channel-group 10 mode auto no sh int port-channel 25 ip add 172.16.1.29 255.255.255.252 no sh int range g1/0/14-15 no switchport channel-group 25 mode active no sh ip helper-address 172.16.1.19 no sh int vlan 160 description Default Gateway SVI for Wlan devices ip add 172.16.160.3 255.255.255.0 standby 160 ip 172.16.160.1 standby 160 priority 200 standby 160 preempt ip helper-address 172.16.1.19 no sh int vlan 170 description Default Gateway SVI for Printer devices ip add 172.16.170.3 255.255.255.0 standby 170 ip 172.16.170.1 standby 170 priority 200 standby 170 preempt ip helper-address 172.16.1.19 no sh - DHCP ip dhcp pool HR network 172.16.10.0 255.255.255.0 default-router 172.16.10.1 dns-server 172.16.5.18 ip dhcp pool Sales network 172.16.20.0 255.255.255.0 default-router 172.16.20.1 dns-server 172.16.5.18 ip dhcp pool IT network 172.16.30.0 255.255.255.0 default-router 172.16.30.1 dns-server 172.16.5.18 ip dhcp pool CEO network 172.16.40.0 255.255.255.0 default-router 172.16.40.1 dns-server 172.16.5.18 ip dhcp pool Finance network 172.16.50.0 255.255.255.0 default-router 172.16.50.1 dns-server 172.16.5.18 ip dhcp pool MGMT network 172.16.100.0 255.255.255.0 default-router 172.16.100.1 dns-server 172.16.5.18 ip dhcp pool Voip network 172.16.150.0 255.255.255.0 default-router 172.16.150.1 dns-server 172.16.5.18 ip dhcp pool Wlan network 172.16.160.0 255.255.255.0 default-router 172.16.160.1 dns-server 172.16.5.18 ip dhcp pool Printer network 172.16.170.0 255.255.255.0 default-router 172.16.170.1 dns-server 172.16.5.18 ip dhcp excluded-address 172.16.10.1 172.16.10.124 ip dhcp excluded-address 172.16.10.200 172.16.10.255 ip dhcp excluded-address 172.16.20.1 172.16.20.124 ip dhcp excluded-address 172.16.20.200 172.16.20.255 ip dhcp excluded-address 172.16.30.1 172.16.30.124 ip dhcp excluded-address 172.16.30.200 172.16.30.255 ip dhcp excluded-address 172.16.40.1 172.16.40.124 ip dhcp excluded-address 172.16.40.200 172.16.40.255 ip dhcp excluded-address 172.16.50.1 172.16.50.124 ip dhcp excluded-address 172.16.50.200 172.16.50.255 ip dhcp excluded-address 172.16.100.1 172.16.100.124 ip dhcp excluded-address 172.16.100.200 172.16.100.255 ip dhcp excluded-address 172.16.150.1 172.16.150.124 ip dhcp excluded-address 172.16.150.200 172.16.150.255 ip dhcp excluded-address 172.16.160.1 172.16.160.124 ip dhcp excluded-address 172.16.160.200 172.16.160.255 ip dhcp excluded-address 172.16.170.1 172.16.170.124 ip dhcp excluded-address 172.16.170.200 172.16.170.255 |
| --- | --- |

HQ_R_1

| - Host name hostname HQ_R_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - Line console line con 0 login authentication AAA - Line VTY (ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 access-class VTY in login authentication AAA transport input ssh line vty 5 10 transport input ssh - STP spanning-tree mode pvst - IP , NAT int GigabitEthernet0/0 no sh ip add 172.16.1.6 255.255.255.252 ip nat inside int GigabitEthernet0/1 no sh ip address 172.16.2.21 255.255.255.252 ip nat inside interface GigabitEthernet0/1/0 ip address dhcp ip nat outside ip nat inside source list 120 interface GigabitEthernet0/1/0 overload | - OSPF , Static Routing router ospf 10 router-id 6.6.6.6 network 172.16.2.20 0.0.0.3 area 0 network 172.16.1.4 0.0.0.3 area 0 network 209.1.34.0 0.0.0.255 area 0 default-information originate ip route 0.0.0.0 0.0.0.0 209.1.34.1 ip route 209.1.12.0 255.255.255.0 209.1.34.1 ip route 172.17.0.0 255.255.0.0 5.0.0.2 - VPN interface Tunnel1 ip address 5.0.0.1 255.255.255.252 mtu 1476 tunnel source GigabitEthernet0/1/0 tunnel destination 209.1.12.8 - ACL access-list 110 permit ip 172.16.0.0 0.0.255.255 172.17.0.0 0.0.255.255 access-list 120 deny ip 172.16.0.0 0.0.255.255 172.17.0.0 0.0.255.255 access-list 120 deny ip 172.16.50.0 0.0.0.255 209.1.0.0 0.0.255.255 access-list 120 permit ip 172.16.0.0 0.0.255.255 any ip access-list extended VTY permit ip 172.16.30.0 0.0.0.255 any permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.40.0 0.0.0.255 any permit ip 172.17.20.0 0.0.0.255 any deny tcp any any - FTP Service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP Service ntp server 172.16.5.35 - Syslog Service logging 172.16.5.35 |
| --- | --- |

HQ_R_2

| - Host name hostname HQ_R_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - Line console line con 0 login authentication AAA - Line VTY (ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 access-class VTY in login authentication AAA transport input ssh line vty 5 10 transport input ssh - STP spanning-tree mode pvst - IP , NAT int GigabitEthernet0/0 no sh ip address 172.16.1.14 255.255.255.252 ip nat inside int GigabitEthernet0/1 no sh ip address 172.16.2.22 255.255.255.252 ip nat inside interface GigabitEthernet0/1/0 interface GigabitEthernet0/1/0 ip address dhcp ip nat outside ip nat inside source list 120 interface GigabitEthernet0/1/0 overload | - OSPF , Static Routing router ospf 10 router-id 5.5.5.5 network 172.16.2.20 0.0.0.3 area 0 network 172.16.1.12 0.0.0.3 area 0 network 209.1.33.0 0.0.0.255 area 0 default-information originate ip route 0.0.0.0 0.0.0.0 209.1.33.1 ip route 209.1.22.0 255.255.255.0 209.1.33.1 ip route 172.17.0.0 255.255.0.0 5.0.0.6 - VPN interface Tunnel1 ip address 5.0.0.5 255.255.255.252 mtu 1476 tunnel source GigabitEthernet0/1/0 tunnel destination 209.1.22.8 - ACL a ip access-list extended VTY permit ip 172.16.30.0 0.0.0.255 any permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.40.0 0.0.0.255 any permit ip 172.17.20.0 0.0.0.255 any deny tcp any any access-list 120 deny ip 172.16.0.0 0.0.255.255 172.17.0.0 0.0.255.255 access-list 120 deny ip 172.16.50.0 0.0.0.255 209.1.0.0 0.0.255.255 access-list 120 permit ip 172.16.0.0 0.0.255.255 any - FTP Service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP Service ntp server 172.16.5.35 - Syslog Service logging 172.16.5.35 |
| --- | --- |

HQ_DC_sw

| - Host name hostname HQ_DC_sw - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Vlans vlan 5 name Zone_1 vlan 15 name Zone_2 vlan 25 name Zone_3 vlan 99 vlan 100 name MGMT name Native vlan 200 name Unused - OSPF router ospf 10 router-id 3.3.3.3 passive-interface FastEthernet0/4 passive-interface FastEthernet0/5 passive-interface FastEthernet0/6 passive-interface FastEthernet0/7 passive-interface FastEthernet0/8 passive-interface FastEthernet0/9 network 172.16.5.16 0.0.0.15 area 0 network 172.16.5.32 0.0.0.15 area 0 network 172.16.5.48 0.0.0.15 area 0 network 172.16.1.28 0.0.0.3 area 0 network 172.16.5.0 0.0.0.15 area 0 - Unused Interfces int range FastEthernet0/10-24 shutdown switchport mode access switchport access vlan 200 - STP bpduguard int range FastEthernet0/4-9 spanning-tree portfast spanning-tree bpduguard - FTP service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 NTP service ntp server 172.16.5.35 Syslog service logging 172.16.5.35 | - Ether-channel int port-channel 5 no switchport ip add 172.16.1.30 255.255.255.252 no sh int range GigabitEthernet0/2-3 no switchport channel-group 5 mode passive no sh - IVLAN interface Vlan5 description Default Gateway SVI for Web & FTP Servers ip address 172.16.5.1 255.255.255.240 interface Vlan15 description Default Gateway SVI for Email & DHCP Servers ip address 172.16.5.17 255.255.255.240 interface Vlan25 description Default Gateway SVI for RADUIS & NTP & Syslog Servers ip address 172.16.5.33 255.255.255.240 interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.1.30 255.255.255.0 - Access Interfaces int range FastEthernet0/4-5 no shutdown switchport mode access switchport access vlan 5 int range FastEthernet0/6-7 no shutdown switchport mode access switchport access vlan 15 int range FastEthernet0/8-9 no shutdown switchport mode access switchport access vlan 25 |
| --- | --- |

HQ_Acc_sw_1

| - Host name hostname HQ_Acc_sw_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.100.21 255.255.255.0 ip default-gateway 172.16.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

HQ_Acc_sw_2

| - Host name hostname HQ_Acc_sw_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.100.22 255.255.255.0 ip default-gateway 172.16.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

HQ_Acc_sw_3

| - Host name hostname HQ_Acc_sw_3 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - Unused Interfaces int range FastEthernet0/6-9 , FastEthernet0/11-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-5 , FastEthernet0/10 spanning-tree portfast - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.100.23 255.255.255.0 ip default-gateway 172.16.100.1 spanning-tree bpduguard enable - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - FTP service ip ftp username devices ip ftp password cisco | spanning-tree bpduguard - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/4 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/5 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/10 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast |
| --- | --- |

HQ_Acc_sw_4

| - Host name hostname HQ_Acc_sw_4 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.100.24 255.255.255.0 ip default-gateway 172.16.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

HQ_Acc_sw_5

| - Host name hostname HQ_Acc_sw_5 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.16.100.25 255.255.255.0 ip default-gateway 172.16.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.16.5.34 auth-port 1645 key Cisco123 radius server 172.16.5.34 address ipv4 172.16.5.34 auth-port 1645 key Cisco123 - NTP service ntp server 172.16.5.35 - Syslog service logging 172.16.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

Head Quarter Data Sheets

  - Routers

Router 1 (Head Quarter)

| Device | Model | Description | Location | IOS |
| --- | --- | --- | --- | --- |
| HQ_R_1 | ISR 2911 | Primary Edge Router | Cairo Branch | Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9-M), Version 15.1(4)M4, RELEASE SOFTWARE (fc2) |
| Interface | Description | IPv4 Address | MAC Address | Routing |
| Port-channel20 | Port-channel | 172.16.1.21/30 | 00d0.d399.e4c0 | N/A |
| GigabitEthernet0/0 | Connected to HQ_DS_sw_1 | 172.16.1.6/30 | 0001.c78c.5401 | OSPF |
| GigabitEthernet0/1 | Channel group 20 | Unassigned | 0001.c78c.5402 | N/A |
| GigabitEthernet0/2 |  |  | 0001.c78c.5403 |  |
| GigabitEthernet0/1/0 | Connected to ISP (WE) | 209.1.34.8/24 | 000a.414d.3461 | N/A |
| Tunnel1 | Connected to Br_R_1 | 5.0.0.1/30 | N/A | N/A |

Router 2 (Head Quarter)

| Device | Model | Description | Location | IOS |
| --- | --- | --- | --- | --- |
| HQ_R_2 | ISR 2911 | Secondary Edge Router | Cairo Branch | Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9-M), Version 15.1(4)M4, RELEASE SOFTWARE (fc2) |
| Interface | Description | IPv4 Address | MAC Address | Routing |
| Port-channel20 | Port-channel | 172.16.1.22/30 | 0010.111a.4567 | N/A |
| GigabitEthernet0/0 | Connected to HQ_DS_sw_2 | 172.16.1.14/30 | 00d0.bcde.2401 | OSPF |
| GigabitEthernet0/1 | Channel group 20 | Unassigned | 00d0.bcde.2402 | N/A |
| GigabitEthernet0/2 |  |  | 00d0.bcde.2403 |  |
| GigabitEthernet0/1/0 | Connected to ISP (Vodafone) | 209.1.33.8/24 | 0005.5e71.b602 | N/A |
| Tunnel1 | Connected to Br_R_2 | 5.0.0.5/30 | N/A | N/A |

1.2 L3 Switches:

L3 Switch 1 (Head Quarter)

| Device |  | Model | Description |  |  | Location |  |  | IOS |  | VTP (V2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_DS_sw_1 |  | 3650-24PS | Primary L3 Switch |  |  | Cairo Branch |  |  | Cisco IOS Software [Denali], Catalyst L3 Switch Software (CAT3K_CAA-UNIVERSALK9-M), Version 16.3.2, RELEASE SOFTWARE (fc4) |  | Domain: Cisco.com Password: Cisco |  |
| Port | Description |  |  | Access | VLAN |  | Trunk | IPv4 Address |  | Native |  | Enabled |
| G 1/0/1 - 4 | Connected to HQ_DS_sw_2 (Channel-group 10) (Pagp) |  |  | - | - |  | - | Unassigned |  | - |  | Yes |
| G 1/0/5 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/6 - 10 | Connected VLANs (50 - 10) |  |  | - | - |  | Yes |  |  | 99 |  | Yes |
| G 1/0/11 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/12 | Connected to HQ_R_2 |  |  | - | - |  | - | 172.16.1.5 /30 |  | - |  | Yes |
| G 1/0/14 - 19 | - |  |  | Yes | 200 |  | - | Unassigned |  | - |  | Shut |
| G 1/0/20 | Connected to HQ_WLC |  |  | - | 200 |  | Yes |  |  | 100 |  | Yes |
| G 1/0/21 - 24 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/1/1 - 4 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
|  |  |  |  |  |  |  |  | - |  |  |  | - |
| VLAN 10 | Connected to HQ_DS_sw_1 & HQ_DS_sw_2 |  |  | - | - |  | - | 172.16.10.2 |  | - |  | Yes |
| VLAN 20 |  |  |  | - | - |  | - | 172.16.20.2 |  | - |  | Yes |
| VLAN 30 |  |  |  | - | - |  | - | 172.16.30.2 |  | - |  | Yes |
| VLAN 40 |  |  |  | - | - |  | - | 172.16.40.2 |  | - |  | Yes |
| VLAN 50 |  |  |  | - | - |  | - | 172.16.50.2 |  | - |  | Yes |
| VLAN 100 |  |  |  | - | - |  | - | 172.16.100.2 |  | - |  | Yes |
| VLAN 150 |  |  |  | - | - |  | - | 172.16.150.2 |  | - |  | Yes |
| VLAN 160 |  |  |  | - | - |  | - | 172.16.160.2 |  | - |  | Yes |
| VLAN 170 |  |  |  | - | - |  | - | 172.16.170.2 |  | - |  | Yes |

L3 Switch 2 (Head Quarter)

| Device |  | Model | Description |  |  | Location |  |  | IOS |  | VTP (V2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_DS_sw_2 |  | 3650-24PS | Secondary L3 Switch |  |  | Cairo Branch |  |  | Cisco IOS Software [Denali], Catalyst L3 Switch Software (CAT3K_CAA-UNIVERSALK9-M), Version 16.3.2, RELEASE SOFTWARE (fc4) |  | Domain: Cisco.com Password: Cisco |  |
| Port | Description |  |  | Access | VLAN |  | Trunk | IPv4 Address |  | Native |  | Enabled |
| G 1/0/1 - 4 | Connected to HQ_DS_sw_1 (Channel-group 10) (PAgP) |  |  | - | - |  | - | Unassigned |  | - |  | Yes |
| G 1/0/5 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/6 - 10 | Connected to VLANs (50 - 10) |  |  | - | - |  | Yes |  |  | 99 |  | Yes |
| G 1/0/11 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/12 | Connected to HQ_R_2 |  |  | - | - |  | - | 172.16.1.13 /30 |  | - |  | Yes |
| G 1/0/14 - 15 | Connected to Data Center Building (Channel-group 25) (LACP) |  |  | - | 200 |  | - | Unassigned |  | - |  | Shut |
| G 1/0/16 - 24 | - |  |  | Yes | 200 |  | - |  |  | 100 |  | Yes |
| G 1/1/1 - 4 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
|  |  |  |  |  |  |  |  | - |  |  |  | - |
| VLAN 10 | Connected to HQ_DS_sw_1 & HQ_DS_sw_2 |  |  | - | - |  | - | 172.16.10.2 |  | - |  | Yes |
| VLAN 20 |  |  |  | - | - |  | - | 172.16.20.2 |  | - |  | Yes |
| VLAN 30 |  |  |  | - | - |  | - | 172.16.30.2 |  | - |  | Yes |
| VLAN 40 |  |  |  | - | - |  | - | 172.16.40.2 |  | - |  | Yes |
| VLAN 50 |  |  |  | - | - |  | - | 172.16.50.2 |  | - |  | Yes |
| VLAN 100 |  |  |  | - | - |  | - | 172.16.100.2 |  | - |  | Yes |
| VLAN 150 |  |  |  | - | - |  | - | 172.16.150.2 |  | - |  | Yes |
| VLAN 160 |  |  |  | - | - |  | - | 172.16.160.2 |  | - |  | Yes |
| VLAN 170 |  |  |  | - | - |  | - | 172.16.170.2 |  | - |  | Yes |

1.3 L2 Switches

L2 Switch 1 (Head Quarter)

| Device |  | Model |  | Description | Mgmt_IP Address |  |  | IOS |  | VTP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_Acc_ sw_1 |  | Cisco Catalyst WS-C2960-24TC-L |  | HQ- switch Floor_1 | 172.16.100.21/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  | Domain: cisco.com Mode: Client |
| interface | Description |  | MAC Address |  |  | Access | VLAN | Trunk | Enable | Native |
| Fa 0/1 | Connects to AP0_1 |  | 0090.2157.8726 |  |  | - | - | Yes | Up | 99 |
| Fa 0/2 | Connects to HQ_PC-1 |  | 000c.cfd4.026a |  |  | Yes | 10 | - | Up | N/A |
| Fa 0/3 | Connects to Printer0_1 |  | 0002.163e.509c |  |  | Yes | 170 | - | Up | N/A |
| G 0/1 | Connects to HQ_DS_sw_1 |  | 0009.7c29.a47a |  |  | - | - | Yes | Up | 99 |
| G 0/2 | Connects to HQ_DS_sw_2 |  | 0002.4a38.9b25 |  |  | - | - | Yes | Up | 99 |
| VLAN 100 | Management Interface |  | 0040.0bd7.c101 |  |  | - | 100 | - | Up | N/A |
| Fa 0/4-24 | Unused |  | N/A |  |  | - | 200 | - | Down | - |

L2 Switch 2 (Head Quarter)

| Device | Model | Description |  | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_Acc_sw_2 | Cisco Catalyst WS-C2960-24TC-L | HQ_switch_Floor_2 |  | 172.16.100.22/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  | MAC Address |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP_2 |  | 00e0.b0ad.a13d |  | - | - |  | Yes | Up |  | 99 |
| Fa 0/2 | Connects to HQ_PC-2 |  | 00d0.d3b8.47ac |  | Yes | 20 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer_2 |  | 0010.117b.eb57 |  | Yes | 170 |  | --- | Up |  | N/A |
| G 0/1 | Connects to HQ_DS_sw_1 |  | 0060.5ce2.337e |  | - | - |  | Yes | Up |  | 99 |
| G 0/2 | Connects to HQ_DS_sw_2 |  | 000d.bd29.1355 |  | - | - |  | Yes | Up |  | 99 |
| VLAN 100 | Management Interface |  | 00d0.ffa1.2101 |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  | N/A |  | - | 200 |  | - | Down |  | - |

L2 Switch 3 (Head Quarter)

| Device | Model |  | Description | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_Acc_sw_3 | Cisco Catalyst WS-C2960-24TC-L |  | HQ_switch_Floor_3 | 172.16.100.23/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description | MAC Address |  |  | Access | VLAN |  | Trunk | Enable |  |  | Native |
| Fa 0/1 | Connects to AP_3 | 0001.c9ed.5a21 |  |  | - | - |  | Yes | Up |  |  | 99 |
| Fa 0/2 | Connects to HQ_PC-3 | 0010.11ba.e394 |  |  | Yes | 30 |  | - | Up |  |  | N/A |
| Fa 0/3 | Connects to Printer_3 | 0001.c774.5810 |  |  | Yes | 170 |  | - | Up |  |  | N/A |
| G 0/1 | Connects to HQ_DS_sw_1 | 00d0.bc36.2a12 |  |  | - | - |  | Yes | Up |  |  | 99 |
| G 0/2 | Connects to HQ_DS_sw_2 | 00e0.f9b4.4d28 |  |  | - | - |  | Yes | Up |  |  | 99 |
| VLAN 100 | Management Interface | 0007.ec7a.7201 |  |  | - | 100 |  | - | Up |  |  | N/A |
| Fa 0/4-24 | Unused | N/A |  |  | - | 200 |  | - | Down |  |  | - |

L2 Switch 4 (Head Quarter)

| Device |  | Model |  | Description | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_Acc_sw_4 |  | Cisco Catalyst WS-C2960-24TC-L |  | HQ- switch Floor_4 | 172.16.100.24/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  | MAC Address |  |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP_4 |  | 00d0.58a2.42bb |  |  | - | - |  | Yes | Up |  | 99 |
| Fa 0/2 | Connects to HQ_PC-4 |  | 0060.701d.4117 |  |  | Yes | 40 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer_4 |  | 0001.43ac.8453 |  |  | Yes | 170 |  | - | Up |  | N/A |
| G 0/1 | Connects to HQ_DS_sw_1 |  | 00e0.f9eb.c213 |  |  | - | - |  | Yes | Up |  | 99 |
| G 0/2 | Connects to HQ_DS_sw_2 |  | 0030.a3ee.d7d4 |  |  | - | - |  | Yes | Up |  | 99 |
| VLAN 100 | Management Interface |  | 0006.2a4a.d801 |  |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  | N/A |  |  | - | 200 |  | - | Down |  | - |

L2 Switch 5 (Head Quarter)

| Device | Model |  | Description | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HQ_Acc_sw_5 | Cisco Catalyst WS-C2960-24TC-L |  | HQ_switch_Floor_5 | 172.16.100.25/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description | MAC Address |  |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP_5 | 00e0.f992.6c08 |  |  | - | - |  | Yes | Up |  | 99 |
| Fa 0/2 | Connects to Br_PC-5 | 0001.c901.5ce9 |  |  | Yes | 50 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer_5 | 0090.0c2c.ad63 |  |  | Yes | 170 |  | - | Up |  | N/A |
| G 0/1 | Connects to HQ_DS_sw_1 | 0002.1619.7ecb |  |  | - | - |  | Yes | Up |  | 99 |
| G 0/2 | Connects to HQ_DS_sw_2 | 000a.41dd.8d70 |  |  | - | - |  | Yes | Up |  | 99 |
| VLAN 100 | Management Interface | 0060.70d8.a701 |  |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused | N/A |  |  | - | 200 |  | - | Down |  | - |

Paris_DS_sw-1

| - Host name hostname Paris_DS_sw_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** Access is Monitored and Logged. ** ** Team Members: ** - Ibrahim Mohammed ** - Ahmed Sabry ** - Shady Hossam ** - Mohamed Nabawy ** - Mohamed Elsayed ** Special Thanks to Our Respected Instructor: ** - Sameh Elshamy ** Have a Great Day! **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Trunk, Native Interfaces int range g1/0/5-11 no sh switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan all int g1/0/12 no switchport ip add 172.17.1.5 255.255.255.252 interface GigabitEthernet1/0/20 switchport access vlan 200 switchport trunk native vlan 100 switchport mode trunk - IVLAV , HSRP , Helper-address int vlan 10 description Default Gateway SVI for PR Team ip add 172.17.10.2 255.255.255.0 standby 10 ip 172.17.10.1 standby 10 priority 200 standby 10 preempt ip helper-address 172.17.1.19 no sh int vlan 20 description Default Gateway SVI for C & S Team ip add 172.17.20.2 255.255.255.0 standby 20 ip 172.17.20.1 standby 20 priority 200 standby 20 preempt ip helper-address 172.17.1.19 no sh int vlan 30 description Default Gateway SVI for SE Team ip add 172.17.30.2 255.255.255.0 standby 30 ip 172.17.30.1 standby 30 priority 200 standby 30 preempt ip helper-address 172.17.1.19 no sh int vlan 100 description Default Gateway SVI for Management Team ip add 172.17.100.2 255.255.255.0 standby 100 ip 172.17.100.1 standby 100 priority 200 standby 100 preempt ip helper-address 172.17.1.19 no sh int vlan 40 description Default Gateway SVI for IT Team ip add 172.17.40.2 255.255.255.0 standby 40 ip 172.17.40.1 standby 40 preempt ip helper-address 172.17.1.19 no sh int vlan 50 description Default Gateway SVI for HR Team ip add 172.17.50.2 255.255.255.0 standby 50 ip 172.17.50.1 standby 50 preempt ip helper-address 172.17.1.19 no sh int vlan 60 description Default Gateway SVI for R & D Team ip add 172.17.60.2 255.255.255.0 standby 40 ip 172.17.60.1 - FTP service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - DHCP ip dhcp pool HR network 172.17.10.0 255.255.255.0 default-router 172.17.10.1 dns-server 172.17.5.18 ip dhcp pool Sales network 172.17.20.0 255.255.255.0 default-router 172.17.20.1 dns-server 172.17.5.18 ip dhcp pool IT network 172.17.30.0 255.255.255.0 default-router 172.17.30.1 dns-server 172.17.5.18 ip dhcp pool CEO network 172.17.40.0 255.255.255.0 default-router 172.17.40.1 dns-server 172.17.5.18 ip dhcp pool Finance network 172.17.50.0 255.255.255.0 default-router 172.17.50.1 dns-server 172.17.5.18 ip dhcp pool MGMT network 172.17.100.0 255.255.255.0 default-router 172.17.100.1 dns-server 172.17.5.18 ip dhcp pool Voip network 172.17.150.0 255.255.255.0 default-router 172.17.150.1 dns-server 172.17.5.18 ip dhcp pool Wlan network 172.17.170.0 255.255.255.0 default-router 172.17.160.1 | - VTP vtp version 2 vtp domain cisco.com vtp mode server vtp password cisco vlan 10 name PR vlan 20 name C & S vlan 30 name SE vlan 40 name IT vlan 50 name HR vlan 60 name R & D vlan 100 name MGMT vlan 150 name VIOP vlan 170 name Wlan vlan 170 name Printer vlan 99 name Native vlan 200 name Unused - STP spanning tree mode pvst spanning tree vlan 10,20,30,40,100 root primary spanning tree vlan 50,60,150,170,170 root secondary - Ether-channel int range g2/0/1-4 no switchport channel-group 1 mode active no sh int port-channel 1 ip add 172.17.1.1 255.255.255.252 switchport trunk native vlan 99 no sh - Unused Interfaces int range g1/0/12-19,g1/0/21-24, g1/1/1-4 sh switchport mode access switchport access vlan 200 standby 60 preempt ip helper-address 172.17.1.19 no sh int vlan 150 description Default Gateway SVI for VIOP devices ip add 172.17.150.2 255.255.255.0 standby 150 ip 172.17.150.1 standby 150 preempt ip helper-address 172.17.1.19 no sh int vlan 160 description Default Gateway SVI for Wlan devices ip add 172.17.170.2 255.255.255.0 standby 170 ip 172.17.170.1 standby 170 preempt ip helper-address 172.17.1.19 no sh int vlan 170 description Default Gateway SVI for Printer devices ip add 172.17.170.2 255.255.255.0 standby 170 ip 172.17.170.1 standby 170 preempt ip helper-address 172.17.1.19 no sh dns-server 172.17.5.18 ip dhcp pool Printer network 172.17.170.0 255.255.255.0 default-router 172.17.170.1 dns-server 172.17.5.18 ip dhcp excluded-address 172.17.10.1 172.17.10.49 ip dhcp excluded-address 172.17.10.125 172.17.10.255 ip dhcp excluded-address 172.17.20.1 172.17.20.49 ip dhcp excluded-address 172.17.20.125 172.17.20.255 ip dhcp excluded-address 172.17.30.1 172.17.30.49 ip dhcp excluded-address 172.17.30.125 172.17.30.255 ip dhcp excluded-address 172.17.40.1 172.17.40.49 ip dhcp excluded-address 172.17.40.125 172.17.40.255 ip dhcp excluded-address 172.17.50.1 172.17.50.49 ip dhcp excluded-address 172.17.50.125 172.17.50.255 ip dhcp excluded-address 172.17.100.1 172.17.100.49 ip dhcp excluded-address 172.17.100.125 172.17.100.255 ip dhcp excluded-address 172.17.150.1 172.17.150.49 ip dhcp excluded-address 172.17.150.125 172.17.150.255 ip dhcp excluded-address 172.17.170.1 172.17.160.49 ip dhcp excluded-address 172.17.170.125 172.17.170.255 ip dhcp excluded-address 172.17.170.1 172.17.170.49 ip dhcp excluded-address 172.17.170.125 172.17.170.255 |
| --- | --- |

Paris_DS_sw_2

| - Host name hostname Paris_DS_sw_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** Track: Cisco Network Administrator ** Group Code: SHR1_ISS2_M1e ** Authorized Personnel Only. ** Access is Monitored and Logged. ** Team Members: ** - Ibrahim Mohammed ** - Ahmed Sabry ** - Shady Hossam ** - Mohamed Nabawy ** - Mohamed Elsayed ** Special Thanks to Our Respected Instructor: ** - Sameh Elshamy ** Have a Great Day! **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - -Trunk Interfaces int range g1/0/5-11 no sh switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan all int g1/0/12 no switchport ip add 172.17.1.13 255.255.255.252 - Unused Interfaces int range g1/0/12-24, g1/1/1-4 sh switchport mode access switchport access vlan 200 - STP spanning tree mode pvst spanning tree vlan 50,60,150,170,170 root primary spanning tree vlan 10,20,30,40,100 root secondary - IVLAV , HSRP , Helper-address int vlan 10 description Default Gateway SVI for PR Team ip add 172.17.10.3 255.255.255.0 standby 10 ip 172.17.10.1 standby 10 preempt ip helper-address 172.17.1.19 no sh int vlan 20 description Default Gateway SVI for C & S Team ip add 172.17.20.3 255.255.255.0 standby 20 ip 172.17.20.1 standby 20 preempt ip helper-address 172.17.1.19 no sh int vlan 30 description Default Gateway SVI for SE Team ip add 172.17.30.3 255.255.255.0 standby 30 ip 172.17.30.1 standby 30 preempt ip helper-address 172.17.1.19 no sh int vlan 100 description Default Gateway SVI for Management Team ip add 172.17.100.3 255.255.255.0 standby 100 ip 172.17.100.1 standby 100 preempt ip helper-address 172.17.1.19 no sh int vlan 40 description Default Gateway SVI for IT Team ip add 172.17.40.3 255.255.255.0 standby 40 ip 172.17.40.1 standby 40 priority 200 standby 40 preempt ip helper-address 172.17.1.19 no sh int vlan 50 description Default Gateway SVI for HR Team ip add 172.17.50.3 255.255.255.0 standby 50 ip 172.17.50.1 standby 50 priority 200 standby 50 preempt ip helper-address 172.17.1.19 no sh - OSPF , Static Routing ip routing router ospf 10 router-id 2.2.2.2 passive-interface GigabitEthernet1/0/6 passive-interface GigabitEthernet1/0/7 passive-interface GigabitEthernet1/0/8 passive-interface GigabitEthernet1/0/9 passive-interface GigabitEthernet1/0/10 passive-interface GigabitEthernet1/0/20 network 172.17.1.0 0.0.0.3 area 1 network 172.17.10.0 0.0.0.255 area 1 network 172.17.20.0 0.0.0.255 area 1 network 172.17.30.0 0.0.0.255 area 1 network 172.17.40.0 0.0.0.255 area 1 network 172.17.50.0 0.0.0.255 area 1 network 172.17.100.0 0.0.0.255 area 1 network 172.17.150.0 0.0.0.255 area 1 network 172.17.160.0 0.0.0.255 area 1 network 172.17.1.12 0.0.0.3 area 1 network 172.17.1.28 0.0.0.3 area 1 network 172.17.170.0 0.0.0.255 area 1 ip route 172.17.5.34 255.255.255.255 172.17.5.30 ip route 172.17.5.35 255.255.255.255 172.17.5.30 - FTP service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 | - VTP vtp version 2 vtp domain cisco.com vtp mode server vtp password cisco vlan 10 name PR vlan 20 name C & S vlan 30 name SE vlan 40 name IT vlan 50 name HR vlan 60 name R & D vlan 100 name MGMT vlan 150 name VIOP vlan 160 name Wlan vlan 170 name Printer vlan 99 name Native vlan 200 - Ether-channel int port-channel 1 ip add 172.17.1.2 255.255.255.252 no sh int range g1/0/2-4 no switchport channel-group 1 mode desirable no sh int vlan 60 description Default Gateway SVI for R & D Team ip add 172.17.50.3 255.255.255.0 standby 50 ip 172.17.50.1 standby 50 priority 200 standby 50 preempt ip helper-address 172.17.1.19 no sh int vlan 150 description Default Gateway SVI for VIOP devices ip add 172.17.150.3 255.255.255.0 standby 150 ip 172.17.150.1 standby 150 priority 200 standby 150 preempt ip helper-address 172.17.1.19 no sh int vlan 160 description Default Gateway SVI for Wlan devices ip add 172.17.170.3 255.255.255.0 standby 170 ip 172.17.170.1 standby 170 priority 200 standby 170 preempt ip helper-address 172.17.1.19 no sh int vlan 170 description Default Gateway SVI for Printer devices ip add 172.17.170.3 255.255.255.0 standby 170 ip 172.17.170.1 standby 170 priority 200 standby 170 preempt ip helper-address 172.17.1.19 no sh - DHCP ip dhcp pool HR network 172.17.10.0 255.255.255.0 default-router 172.17.10.1 dns-server 172.17.5.18 ip dhcp pool Sales network 172.17.20.0 255.255.255.0 default-router 172.17.20.1 dns-server 172.17.5.18 ip dhcp pool IT network 172.17.30.0 255.255.255.0 default-router 172.17.30.1 dns-server 172.17.5.18 ip dhcp pool CEO network 172.17.40.0 255.255.255.0 default-router 172.17.40.1 dns-server 172.17.5.18 ip dhcp pool Finance network 172.17.50.0 255.255.255.0 default-router 172.17.50.1 dns-server 172.17.5.18 ip dhcp pool MGMT network 172.17.100.0 255.255.255.0 default-router 172.17.100.1 dns-server 172.17.5.18 ip dhcp pool Voip network 172.17.150.0 255.255.255.0 default-router 172.17.150.1 dns-server 172.17.5.18 ip dhcp pool Wlan network 172.17.170.0 255.255.255.0 default-router 172.17.160.1 dns-server 172.17.5.18 ip dhcp pool Printer network 172.17.170.0 255.255.255.0 default-router 172.17.170.1 dns-server 172.17.5.18 ip dhcp excluded-address 172.17.10.1 172.17.10.124 ip dhcp excluded-address 172.17.10.200 172.17.10.255 ip dhcp excluded-address 172.17.20.1 172.17.20.124 ip dhcp excluded-address 172.17.20.200 172.17.20.255 ip dhcp excluded-address 172.17.30.1 172.17.30.124 ip dhcp excluded-address 172.17.30.200 172.17.30.255 ip dhcp excluded-address 172.17.40.1 172.17.40.124 ip dhcp excluded-address 172.17.40.200 172.17.40.255 ip dhcp excluded-address 172.17.50.1 172.17.50.124 ip dhcp excluded-address 172.17.50.200 172.17.50.255 ip dhcp excluded-address 172.17.100.1 172.17.100.124 ip dhcp excluded-address 172.17.100.200 172.17.100.255 ip dhcp excluded-address 172.17.150.1 172.17.150.124 ip dhcp excluded-address 172.17.150.200 172.17.150.255 ip dhcp excluded-address 172.17.170.1 172.17.160.124 ip dhcp excluded-address 172.17.170.200 172.17.170.255 ip dhcp excluded-address 172.17.170.1 172.17.170.124 ip dhcp excluded-address 172.17.170.200 172.17.170.255 |
| --- | --- |

Paris_R_1

| - Host name hostname Paris_R_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - Line console line con 0 login authentication AAA - Line VTY (ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 access-class VTY in login authentication AAA transport input ssh line vty 5 10 transport input ssh - STP spanning-tree mode pvst - IP , NAT int GigabitEthernet0/0 no sh ip add 172.17.1.6 255.255.255.252 ip nat inside int GigabitEthernet0/1 no sh ip address 172.17.2.21 255.255.255.252 ip nat inside interface GigabitEthernet0/1/0 ip address dhcp ip nat outside ip nat inside source list 120 interface GigabitEthernet0/1/0 overload | - OSPF , Static Routing router ospf 10 router-id 6.6.6.6 network 172.17.2.20 0.0.0.3 area 1 network 172.17.1.4 0.0.0.3 area 1 network 209.1.34.0 0.0.0.255 area 1 default-information originate ip route 0.0.0.0 0.0.0.0 209.1.34.1 ip route 209.1.12.0 255.255.255.0 209.1.34.1 ip route 172.17.0.0 255.255.0.0 5.0.0.2 - VPN interface Tunnel1 ip address 5.0.0.2 255.255.255.252 mtu 1476 tunnel source GigabitEthernet0/1/0 tunnel destination 209.1.34.8 - ACL access-list 1 permit 172.17.0.0 0.0.255.255 ip access-list extended VTY permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.40.0 0.0.0.255 any permit ip 172.17.20.0 0.0.0.255 any deny tcp any any - FTP Service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP Service ntp server 172.17.5.35 - Syslog Service logging 172.17.5.35 |
| --- | --- |

Paris_R_2

| - Host name hostname Paris_R_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - Line console line con 0 login authentication AAA - Line VTY (ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 access-class VTY in login authentication AAA transport input ssh line vty 5 10 transport input ssh - STP spanning-tree mode pvst - IP , NAT int GigabitEthernet0/0 no sh ip address 172.17.1.14 255.255.255.252 ip nat inside int GigabitEthernet0/1 no sh ip address 172.17.2.22 255.255.255.252 ip nat inside interface GigabitEthernet0/1/0 interface GigabitEthernet0/1/0 ip address dhcp ip nat outside ip nat inside source list 1 interface GigabitEthernet0/1/0 overload | - OSPF , Static Routing router ospf 10 router-id 5.5.5.5 network 172.17.2.20 0.0.0.3 area 0 network 172.17.1.12 0.0.0.3 area 0 network 209.1.33.0 0.0.0.255 area 0 default-information originate ip route 0.0.0.0 0.0.0.0 209.1.33.1 ip route 209.1.22.0 255.255.255.0 209.1.33.1 ip route 172.17.0.0 255.255.0.0 5.0.0.6 - VPN interface Tunnel1 ip address 5.0.0.6 255.255.255.252 mtu 1476 tunnel source GigabitEthernet0/1/0 tunnel destination 209.1.33.8 - ACL a ip access-list extended VTY permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.30.0 0.0.0.255 any permit ip 172.17.40.0 0.0.0.255 any permit ip 172.17.20.0 0.0.0.255 any deny tcp any any access-list 1 permit 172.17.0.0 0.0.255.255 - FTP Service ip ftp username devices ip ftp password cisco - AAA Service aaa authentication login AAA group radius radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP Service ntp server 172.17.5.35 - Syslog Service logging 172.17.5.35 |
| --- | --- |

Paris_Acc_sw_1

| Host name hostname Paris_Acc_sw_1 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.21 255.255.255.0 ip default-gateway 172.17.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

PARIS_Acc_sw_2

| - Host name hostname PARIS_Acc_sw_2 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.22 255.255.255.0 ip default-gateway 172.17.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

PARIS_Acc_sw_3

| - Host name hostname PARIS_Acc_sw_3 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - Unused Interfaces int range FastEthernet0/6-9 , FastEthernet0/11-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-5 , FastEthernet0/10 spanning-tree portfast - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.23 255.255.255.0 ip default-gateway 172.17.100.1 spanning-tree bpduguard enable - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco | spanning-tree bpduguard - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/4 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/5 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable interface FastEthernet0/10 switchport access vlan 30 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast |
| --- | --- |

PARIS_Acc_sw_4

| - Host name hostname PARIS_Acc_sw_4 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.24 255.255.255.0 ip default-gateway 172.17.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

PARIS_Acc_sw_5

| - Host name hostname PARIS_Acc_sw_5 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.25 255.255.255.0 ip default-gateway 172.17.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

PARIS_Acc_sw_6

| - Host name hostname PARIS_Acc_sw_6 - Banner motd banner motd $ **************************************************** ** Welcome to the Full Secure Campus Network! ** ** Track: Cisco Network Administrator ** ** Group Code: SHR1_ISS2_M1e ** ** Authorized Personnel Only. ** ** Access is Monitored and Logged. ** ** Team Members: ** ** - Ibrahim Mohammed ** ** - Ahmed Sabry ** ** - Shady Hossam ** ** - Mohamed Nabawy ** ** - Mohamed Elsayed ** ** Special Thanks to Our Respected Instructor: ** ** - Sameh Elshamy ** ** Have a Great Day! ** **************************************************** $ - line console line con 0 login authentication AAA - Line console(ssh) crypto key generate rsa general-key modulus 1024 ip ssh version 2 line vty 0 4 login authentication AAA transport input ssh line vty 5 10 login authentication AAA transport input ssh - Unused Interfaces int range FastEthernet0/4-24 shutdown switchport mode access switchport access vlan 200 - STP spanning-tree mode pvst int range FastEthernet0/1-3 spanning-tree portfast spanning-tree bpduguard - MGMT Interface interface Vlan100 description Default Gateway SVI for Management Team ip address 172.17.100.25 255.255.255.0 ip default-gateway 172.17.100.1 - VTP vtp version 2 vtp domain cisco.com vtp mode client vtp password cisco | - L2 Security int FastEthernet0/1 switchport mode atrunk switchport trunk native vlan 100 spanning-tree portfast interface FastEthernet0/2 switchport access vlan 10 switchport mode access switchport nonegotiate switchport voice vlan 150 switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable mls qos trust cos mls qos trust device cisco-phone interface FastEthernet0/3 switchport access vlan 170 switchport mode access switchport nonegotiate switchport port-security maximum 2 switchport port-security mac-address sticky spanning-tree portfast spanning-tree bpduguard enable - Trunk , Native Interfaces int range GigabitEthernet0/1-2 no shutdown switchport mode trunk switchport nonegotiate switchport trunk native vlan 99 switchport trunk allowed vlan all - AAA Service aaa authentication login AAA group radius radius server RADIUS address ipv4 172.17.5.34 auth-port 1745 key Cisco123 radius server 172.17.5.34 address ipv4 172.17.5.34 auth-port 1745 key Cisco123 - NTP service ntp server 172.17.5.35 - Syslog service logging 172.17.5.35 - FTP service ip ftp username devices ip ftp password cisco |
| --- | --- |

Paris Branch Data Sheets

  - Routers

Router 1 (Paris Branch)

| Device | Model | Description | Location | IOS |
| --- | --- | --- | --- | --- |
| Br_R_1 | ISR 2911 | Primary Edge Router | Paris Branch | Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9-M), Version 15.1(4)M4, RELEASE SOFTWARE (fc2) |
| Interface | Description | IPv4 Address | MAC Address | Routing |
| Port-channel20 | Port-channel | 172.17.1.21/30 | 00e0.8fd7.d28c | N/A |
| GigabitEthernet0/0 | Connected to Br_DR_sw_1 | 172.17.1.6/30 | 0030.f29a.0901 | OSPF |
| GigabitEthernet0/1 | Channel group 20 | Unassigned | 0030.f29a.0902 | N/A |
| GigabitEthernet0/2 |  |  | 0030.f29a.0903 |  |
| GigabitEthernet0/1/0 | Connected to ISP (WE) | 209.1.12.8/24 | 000a.f36d.2385 | N/A |
| Tunnel1 | Connected to HQ_R_1 | 5.0.0.2/30 | N/A | N/A |

Router 2 (Paris Branch)

| Device | Model | Description | Location | IOS |
| --- | --- | --- | --- | --- |
| Br_R_2 | ISR 2911 | Secondary Edge Router | Paris Branch | Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9-M), Version 15.1(4)M4, RELEASE SOFTWARE (fc2) |
| Interface | Description | IPv4 Address | MAC Address | Routing |
| Port-channel20 | Port-channel | 172.17.1.22/30 | 0030.a3d3.1e1a | N/A |
| GigabitEthernet0/0 | Connected to Br_DS_sw_2 | 172.17.1.14/30 | 0090.2b30.9701 | OSPF |
| GigabitEthernet0/1 | Channel group 20 | Unassigned | 0090.2b30.9702 | N/A |
| GigabitEthernet0/2 |  |  | 0090.2b30.9703 |  |
| GigabitEthernet0/1/0 | Connected to ISP (Vodafone) | 209.1.22.8/24 | 0090.0c43.6291 | N/A |
| Tunnel1 | Connected to HQ_R_2 | 5.0.0.6/30 | N/A | N/A |

  - L3 Switches

L3 Switch 1 (Paris Branch)

| Device |  | Model | Description |  |  | Location |  |  | IOS |  | VTP (V2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_DS_sw_1 |  | 3650-24PS | Primary L3 Switch |  |  | Paris Branch |  |  | Cisco IOS Software [Denali], Catalyst L3 Switch Software (CAT3K_CAA-UNIVERSALK9-M), Version 16.3.2, RELEASE SOFTWARE (fc4) |  | Domain: Cisco.com Password: Cisco |  |
| Port | Description |  |  | Access | VLAN |  | Trunk | IPv4 Address |  | Native |  | Enabled |
| G 1/0/1 - 4 | Connected to Br_DS_sw_2 (Channel-group 1) (Pagp) |  |  | - | - |  | - | Unassigned |  | - |  | Yes |
| G 1/0/5 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/6 - 11 | Connected VLANs (60 - 10) |  |  | - | - |  | Yes |  |  | 99 |  | Yes |
| G 1/0/12 | Connected to Br_R_2 |  |  | - | - |  | - | 172.17.1.5 /30 |  | - |  | Yes |
| G 1/0/14 - 19 | - |  |  | Yes | 200 |  | - | Unassigned |  | - |  | Shut |
| G 1/0/20 | Connected to Br_WLC |  |  | - | 200 |  | Yes |  |  | 100 |  | Yes |
| G 1/0/21 - 24 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/1/1 - 4 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
|  |  |  |  |  |  |  |  | - |  |  |  | - |
| VLAN 10 | Connected to Br_DS_sw_1 & Br_DS_sw_2 |  |  | - | - |  | - | 172.17.10.2 |  | - |  | Yes |
| VLAN 20 |  |  |  | - | - |  | - | 172.17.20.2 |  | - |  | Yes |
| VLAN 30 |  |  |  | - | - |  | - | 172.17.30.2 |  | - |  | Yes |
| VLAN 40 |  |  |  | - | - |  | - | 172.17.40.2 |  | - |  | Yes |
| VLAN 50 |  |  |  | - | - |  | - | 172.17.50.2 |  | - |  | Yes |
| VLAN 60 |  |  |  | - | - |  | - | 172.17.60.2 |  | - |  | Yes |
| VLAN 100 |  |  |  | - | - |  | - | 172.17.100.2 |  | - |  | Yes |
| VLAN 150 |  |  |  | - | - |  | - | 172.17.150.2 |  | - |  | Yes |
| VLAN 160 |  |  |  | - | - |  | - | 172.17.160.2 |  | - |  | Yes |
| VLAN 170 |  |  |  | - | - |  | - | 172.17.170.2 |  |  |  | Yes |

L3 Switch 2 (Paris Branch)

| Device |  | Model | Description |  |  | Location |  |  | IOS |  | VTP (V2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_DS_sw_2 |  | 3650-24PS | Secondary L3 Switch |  |  | Paris Branch |  |  | Cisco IOS Software [Denali], Catalyst L3 Switch Software (CAT3K_CAA-UNIVERSALK9-M), Version 16.3.2, RELEASE SOFTWARE (fc4) |  | Domain: Cisco.com Password: Cisco |  |
| Port | Description |  |  | Access | VLAN |  | Trunk | IPv4 Address |  | Native |  | Enabled |
| G 1/0/1 - 4 | Connected to Br_DS_sw_1 (Channel-group 1) (PAgP) |  |  | - | - |  | - | Unassigned |  | - |  | Yes |
| G 1/0/5 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
| G 1/0/6 - 11 | Connected to VLANs (60 - 10) |  |  | - | - |  | Yes |  |  | 99 |  | Yes |
| G 1/0/12 | Connected to Br_R_2 |  |  | - | - |  | - | 172.17.1.13 /30 |  | - |  | Yes |
| G 1/0/14 - 15 | - |  |  | - | 200 |  | - | Unassigned |  | - |  | Shut |
| G 1/0/16 - 24 | - |  |  | Yes | 200 |  | - |  |  | 100 |  | Yes |
| G 1/1/1 - 4 | - |  |  | Yes | 200 |  | - |  |  | - |  | Shut |
|  |  |  |  |  |  |  |  | - |  |  |  | - |
| VLAN 10 | Connected to Br_DS_sw_1 & Br_DS_sw_2 |  |  | - | - |  | - | 172.17.10.2 |  | - |  | Yes |
| VLAN 20 |  |  |  | - | - |  | - | 172.17.20.2 |  | - |  | Yes |
| VLAN 30 |  |  |  | - | - |  | - | 172.17.30.2 |  | - |  | Yes |
| VLAN 40 |  |  |  | - | - |  | - | 172.17.40.2 |  | - |  | Yes |
| VLAN 50 |  |  |  | - | - |  | - | 172.17.50.2 |  | - |  | Yes |
| VLAN 100 |  |  |  | - | - |  | - | 172.17.100.2 |  | - |  | Yes |
| VLAN 150 |  |  |  | - | - |  | - | 172.17.150.2 |  | - |  | Yes |
| VLAN 160 |  |  |  | - | - |  | - | 172.17.160.2 |  | - |  | Yes |
| VLAN 170 |  |  |  | - | - |  | - | 172.17.170.2 |  | - |  | Yes |

  - L2 Switches

L2 Switch 1 (Paris Branch)

| Device |  | Model |  | Description | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_1 |  | Cisco Catalyst WS-C2960-24TC-L |  | Branch_switch_Floor_1 | 172.17.100.21/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  | MAC Address |  |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP0_1 |  | 0002.4a67.8201 |  |  | - | - |  | Yes | Up |  | N/A |
| Fa 0/2 | Connects to Br_PC-1 |  | 0002.4a67.8202 |  |  | Yes | 10 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer0_1 |  | 0002.4a67.8203 |  |  | Yes | 170 |  | - | Up |  | N/A |
| G 0/1 | Connects to Br_DS_sw_1 |  | 0002.4a67.8219 |  |  | - | - |  | Yes | Up |  | N/A |
| G 0/2 | Connects to Br_DS_sw_2 |  | 0002.4a67.821a |  |  | - | - |  | Yes | Up |  | N/A |
| VLAN 100 | Management Interface |  | 0060.4793.3601 |  |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  | N/A |  |  | - | 200 |  | - | Down |  | - |

L2 Switch 2 (Paris Branch)

| Device |  | Model | Description | Mgmt_IP Address |  | IOS |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_2 |  | Cisco Catalyst WS-C2960-24TC-L | Branch_switch_Floor_2 | 172.17.100.22/24 |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  | MAC Address | Access | VLAN | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP0_2 |  | 0001.c9aa.b7c9 | - | - | Yes | Up |  | 99 |
| Fa 0/2 | Connects to Br_PC-2 |  | 0001.63ab.5502 | Yes | 20 | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer0_2 |  | 0005.5ea5.b3ee | Yes | 170 | - | Up |  | N/A |
| G 0/1 | Connects to Br_DS_sw_1 |  | 0050.0f98.ee12 | - | - | Yes | Up |  | 99 |
| G 0/2 | Connects to Br_DS_sw_2 |  | 0060.4797.bbde | - | - | Yes | Up |  | 99 |
| VLAN 100 | Management Interface |  | 00d0.d343.5101 | - | 100 | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  | N/A | - | 200 | - | Down |  | - |

L2 Switch 3 (Paris Branch)

| Device | Model | Description | Mgmt_IP Address |  | IOS |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_3 | Cisco Catalyst WS-C2960-24TC-L | Branch_switch_Floor_3 | 172.17.100.23/24 |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  | Domain: cisco.com Mode: Client |  |
| interface | Description | MAC Address | Access | VLAN | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP0_3 | 0009.7c6b.6580 | - | - | Yes | Up |  | 99 |
| Fa 0/2 | Connects to Br_PC-3 | 0001.965c.2a98 | Yes | 30 | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer0_3 | 0004.9a76.bde8 | Yes | 170 | - | Up |  | N/A |
| G 0/1 | Connects to Br_DS_sw_1 | 0090.2b9b.6e14 | - | - | Yes | Up |  | 99 |
| G 0/2 | Connects to Br_DS_sw_2 | 0010.11b4.b885 | - | - | Yes | Up |  | 99 |
| VLAN 100 | Management Interface | 0007.ec92.1801 | - | 100 | - | Up |  | N/A |
| Fa 0/4-24 | Unused | N/A | - | 200 | - | Down |  | - |

L2 Switch 4 (Paris Branch)

| Device | Model | Description | Mgmt_IP Address |  | IOS |  | VTP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_4 | Cisco Catalyst WS-C2960-24TC-L | Branch- switch -Floor_4 | 172.17.100.24/24 |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  | Domain: cisco.com Mode: Client |
| interface | Description | MAC Address | Access | VLAN | Trunk | Enable | Native |
| Fa 0/1 | Connects to AP0_4 | 000c.85e4.4b3c | - | - | Yes | Up | 99 |
| Fa 0/2 | Connects to Br_PC-4 | 000a.418d.95bb | Yes | 40 | - | Up | N/A |
| Fa 0/3 | Connects to Printer0_4 | 0001.c94b.c09e | Yes | 170 | - | Up | N/A |
| G 0/1 | Connects to Br_DS_sw_1 | 0001.64c0.e405 | - | - | Yes | Up | 99 |
| G 0/2 | Connects to Br_DS_sw_2 | 0009.7c8b.e9e9 | - | - | Yes | Up | 99 |
| VLAN 100 | Management Interface | 00e0.f7c3.da01 | - | 100 | - | Up | N/A |
| Fa 0/4-24 | Unused | N/A | - | 200 | - | Down | - |

L2 Switch 5 (Paris Branch)

| Device |  | Model | Description |  | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_5 |  | Cisco Catalyst WS-C2960-24TC-L | Branch_switch_Floor_5 |  | 172.17.100.25/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  |  | MAC Address |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP0_5 |  |  | 00e0.f992.6c08 |  | - | - |  | Yes | Up |  | 99 |
| Fa 0/2 | Connects to Br_PC-5 |  |  | 0001.c901.5ce9 |  | Yes | 50 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer0_5 |  |  | 0090.0c2c.ad63 |  | Yes | 170 |  | - | Up |  | N/A |
| G 0/1 | Connects to Br_DS_sw_1 |  |  | 0002.1619.7ecb |  | - | - |  | Yes | Up |  | 99 |
| G 0/2 | Connects to Br_DS_sw_2 |  |  | 000a.41dd.8d70 |  | - | - |  | Yes | Up |  | 99 |
| VLAN 100 | Management Interface |  |  | 0060.70d8.a701 |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  |  | N/A |  | - | 200 |  | - | Down |  | - |

L2 Switch 6 (Paris Branch)

| Device |  | Model |  | Description | Mgmt_IP Address |  |  | IOS |  |  | VTP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Br_Acc_sw_6 |  | Cisco Catalyst WS-C2960-24TC-L |  | Branch_switch_Floor_6 | 172.17.100.26/24 |  |  | Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4 |  |  | Domain: cisco.com Mode: Client |  |
| interface | Description |  | MAC Address |  |  | Access | VLAN |  | Trunk | Enable |  | Native |
| Fa 0/1 | Connects to AP0_6 |  | 0001.4223.93b0 |  |  | - | - |  | Yes | Up |  | 99 |
| Fa 0/2 | Connects to Br_PC-6 |  | 0090.2b2c.6ab8 |  |  | Yes | 60 |  | - | Up |  | N/A |
| Fa 0/3 | Connects to Printer0_6 |  | 0060.3ea6.4186 |  |  | Yes | 170 |  | - | Up |  | N/A |
| G 0/1 | Connects to Br_DS_sw_1 |  | 0090.0c29.e3c8 |  |  | - | - |  | Yes | Up |  | 99 |
| G 0/2 | Connects to Br_DS_sw_2 |  | 00d0.9769.2441 |  |  | - | - |  | Yes | Up |  | 99 |
| VLAN 100 | Management Interface |  | 0040.0b10.4a01 |  |  | - | 100 |  | - | Up |  | N/A |
| Fa 0/4-24 | Unused |  | N/A |  |  | - | 200 |  | - | Down |  | - |

## Conclusion

This documentation outlines the comprehensive setup undertaken to interconnect the Cairo and Paris branches of our company. By implementing robust VLAN configurations, dual ISP connectivity, and secure VPN tunnels, we have established a reliable and secure network infrastructure. The collaborative efforts of the team members have ensured the successful completion of server configurations, network security implementations, and advanced network functionalities. This setup not only enhances communication and resource sharing between the branches but also ensures scalability and security for future expansions.